Read me of windows robocup base source

By huqian

1, Preface	1

2, How to build and run released agents with log	1

3, How read StepLog and debug line by line	2

4, Are all of decisions under OFFLINE_READ the same as the commands which online server received.	5

# 1, Preface

Robocup is so attractive to me, so I plan to learn to enter the area and make a team if possible. Thanks very much for akiyama-san’s help. I use the open source from him (http://sourceforge.jp/projects/rctools/files/).

Because nearly up to 95% of my program practice is under windows, I am not able to program under linux. So that is why I changes akiyama-san’s open source to windows. As well there is also another reason, that is, I want to use breakpoint to stop agent, and find out why them made such a decision.

Here I am going to explain how to do and how it works.

# 2, How to build and run released agents with log

There are several steps to go till the agents can run with server.

Step1, I use Microsoft Visual Studio 2005 to edit the source. You may double-click base.sln to open the solution. There are 5 types of build mode.

Now choose “Release With StepLog” and build. (Some warnings may show during building)

Notice: If you choose “Relase Without StepLog”, the agent will run without any log file.

Step 2, Open folder ./ robocup_base_070912/bin , You can find base.exe if the building process finished successfully. There is another file named “start_all.bat”, which is a launch script under windows. Before you start your agent, you should start robocup server first. (You can found server here). Then double-click start_all.bat. 11 agents will be launched as command applications.

Step 3, kick off to start match.(kick off has to be done by soccer moniter). The agent will not stop until the game finishes. Perhaps you have noticed that the agent created several file (*.ilog, *.dlog). Right, each agent creates 2 files to log his status at online mode. I call the log “StepLog”.

# 3, How read StepLog and debug line by line

Step 1, Now you have to rebuild solution with “Debug OFF_LINE” read mode.

Then start debug (press F5)

Step 2, A dialog will show to ask you select one ilog file. (Each ilog means one agent log). Now select one and open it.

Step 3, If log file is read successfully, you can see the main dialog. The left of dialog show how many loops the log has. Select one loop and click “Run One Loop” or double click the list item. The agent will do decision by the log, the result will show in edit box at left-down.

Step 4, Now you are able to place breakpoint and run one loop. Then VS2005 will stop at your breakpoint. You can see everything you want.

# 4, Are all of decisions under OFFLINE_READ the same as the commands which online server received.

Now you may load rcl file to check it. (rcl file is the robocup server command file)

Step 1, Click the button “Load rcl file” at the right-down. Select rcl file.

Step 2, A dialog opens to ask you to select one player. You have select the right player according to your steplog file. (For example, if you selected StepDebug_11.ilog in initial dialog, you have to choose num 11 this time.) Then click OK.

Step 3, Now run one loop by double-click left list item. Then the command which online server received, will show in the right-bottom edit box. If the command is the same as decision, the static label in the center shows “==”. If not, it shows “!=”.

Notice: Some loops have lots of index, such as Loop 0, the goal Loop. In those loops, it may be “!=”.